CASWELL

electroplating in miniature

4336 Route 31	Voice – (315) 597-5140

Palmyra	Voice – (315) 597-6378

NY 14522-9719	Fax – (315) 597-1457

USA	http://www.caswellplating.com

sales@caswellplating.com

Date prepared: March 16th 2009

1. Product Identification

Deep Black Anodizing Dye

Chemical family: Metal complex dyestuff

Chemical name: Proprietary

2. Hazardous components of mixture None as per 29CFR Part 1910.1200

Ingredients:

*Chromium (III): <5.0% 	*Formic acid: <3.0%	*Hexylene glycol: <3.0%

*Ingredients reportable under section 313.

3. Physical data

Appearance: Black paste/liquid

Odor: Mild

Solubility in water: Complete

4. Fire and explosion

Extinguishing media: Water, CO2, dry chemical

Unusual fire/explosion hazards: None, not a fire hazard.

5. Reactivity hazards

Hazardous decomposition products: Burning will produce oxides of carbon, nitrogen and/or sulfur.

Stability: Stable under normal conditions.

Hazardous polymerization: Will not occur.

6. Health Hazards

Routes and indication of exposure:

Inhalation: May cause respiratory irritation.

Ingestion: May cause gastric disturbances and nausea.

Skin: May cause irritation to skin.

Eyes: Contact can cause moderate to severe irritation and possible injury to eyes.

7. Accidental release and disposal

Steps to be taken in case material is released or spilled: Wear protective equipment.  Neutralize spilled materials(absorb, pump, vacuum, shovel) immediately.  Prevent run off to drains, sewers, streams or other public waterways.  Follow all precautions for handling this product when dealing with spills.

Waste disposal method: Waste material may be dumped under conditions that meet all Federal, State and local environmental control regulations.

Empty containers: Residues of this product may remain in emptied containers.  Containers should not be used without professional cleaning and reconditioning.  Empty containers should be rinsed before disposal.

Follow all precautions on label when dealing with empty containers.

8. Personal protective equipment

Respiratory protection: An appropriate NIOSH approved respirator for vapors should be worn.

Ventilation: Use of local ventilation is suggested when using this product.

Eye/face protection: Wear chemical goggles, face shields(if splashing is possible).

Skin protection: Wear impermeable rubber or plastic gloves.  wear a rubber or plastic apron.  Wear normal work clothes, long sleeved shirt and pants and clean after every shift.

Personal protective equipment: Eye wash fountain and emergency showers are recommended.

Good industrial practice: In accordance with good industrial practice, avoid all personal contact.  For good hygiene, a shower at the end of every shift is recommended. Wash hands well before eating.

9. First aid measures

Inhalation: If dust or vapors of this product is inhaled, remove to fresh air.  Get medical attention if symptoms develop or persist.

Ingestion: Dilute with water and  induce vomiting and get  medical attention.  Never give fluids if patient is unconscious.

Skin: Wash skin contact with soap and water.  If irritation develops, get medical attention.

Eyes: Immediately flush eye contact with large amounts of water for at least 15 minutes and get medical attention.

10. Exposure control

Engineering controls: Use general ventilation and use local exhaust where possible in confined spaces.

11. Toxicological

Acute toxicity:  No specific acute hazards are known to us.  However, any material that get into eyes or on skin can be irritating.

12. Environmental

Heavy metals toxic to sludge organisms and fish.

13. Transportation

DOT hazard classification: Not regulated.

14. HMIS ratings

Health: 			0	Fire: 			1	Reactivity: 		0

Personal protection:		E

15. Storage

Keep container tightly closed in a dry place away from excessive heat.  Avoid unnecessary personal contact.